OC/SURVEY

RECORD OF SURVEY MAP CHECKING CHECK LIST

RS No.__________________

□  City of ____________________            □   Unincorporated Territory

1st check by:   ____________________________

2nd check by:  ____________________________

linen check by:  ___________________________

Fees paid if monumenting LLA or for future subdivision   ________

THE FOLLOWING MUST APPEAR AT THE TOP OF ALL SHEETS:

□  Map Number

□  Date of Survey

□  Surveyor’s/Engineer’s name and license number (Firm name may be shown)

□  Sheet __ of __

THE FOLLOWING MUST APPEAR ON THE TITLE SHEET:

□  Legal description near the top center, to include the words:

□ “In the Unincorporated Territory of the County of Orange, State of California”

OR

□ “In the City of _________, County of Orange, State of California”

Note: The City, County, and State words must be on ALL sheets, the legal

description can, but does not need to, be shown on all sheets.

□  Surveyor’s/ Engineer’s Statement (must be signed and sealed)

□  County Surveyor’s Statement

□  Clerk - Recorder’s Certificate (Top right corner only – Recorder requirement)

□  Statement of Purpose

THE FOLLOWING MUST APPEAR ON ALL MAP SHEETS:

□  Map must be legible – including size of fonts/letters (minimum text size is 0.08’’)

□  North Arrow

□  Scale

□  References (legible) to adjoining maps or other records as pertinent.

□  In a different font, preferably ghosted

□  Relation of each map sheet to adjoining sheet(s) clearly shown.

□  Basis of Bearings Note  (or on one sheet with reference notes on other sheets)

□  Monument Note on each sheet (or on one sheet with reference notes on other sheets)

□  All found monuments must be individually described or referenced to this note.

□  All monuments must show a record reference or state “No Reference”

MONUMENTATION:

□  Found monuments should use solid symbols

□  Monuments found without tags, and used for boundary control, shall be tagged

MAPPING STANDARDS:

□  Map size:  18” x 26” with 1” border on all sides

□  North Arrow directed away from the reader. (map should be readable from the

bottom (South) and right side (East)

□  Exterior boundary line shall be a solid line at least 3 times heavier than any other line.

□  The map boundary must be tied to adjoining street centerlines in two places, if

applicable.

□  All streets must show the half widths and full width.

□  City/County boundary labeled (if applicable)

□  Symbols and non-standard abbreviations must be defined

SURVEYING PROCEDURES:

Methodology:

□  Survey based on proper control

□  Distinctive boundary property established

□  Prorations/Proportions correct

□  Sectional breakdowns correct

□  Deed interpretations correct

□  Relationship to adjacent lines of record (when pertinent)

□  Methods of establishment of lines or points noted where necessary

Miscellaneous:

□  All reverse or compound curves labeled accordingly (PRC, PCC), with radial bearings.

□  All radial bearings shown where non-tangent curves are involved.

□  All curve data shown

□  All bearings and distances shown

□  All overall bearings and distances shown

□  Sum of parts equals total distance or delta

□  Record data shown in parentheses, with the appropriate reference.

□  Dimension arrows clearly defined

□  Detail provided as needed

□  Spelling correct

□  Traverse(s) close within a reasonable tolerance (0.00’ – 0.017’)

□  Traverse(s) closures match the map

GPS:  not required, but if used . . .

□  GPS descriptions, coordinates, and grid inverse data verified. If using GPS points for cadastral

positions (street C/L etc) a cadastral reference must be used (i.e. TR.  PMB,  RSB)

□  GPS Basis of Bearings line labeled. GPS Tie lines shall be labeled as “Tie”.

□  GPS Tie lines are to distinctive boundary.

□  GPS Tie lines verified as to property position.  [land base or Vmap]

If the map is subdividing property   (8762.5)

□

□

□